SETUP version 2.1 by Kenneth Herron

Notes:

1) This program is in the public domain.  The version upon which

it is based was originally placed in the Public Domain by Bob

Kamins and Tyler Ivanco.

2) The hard disk descriptions are generated by reading the table

in ROM, and so some entries may be different from machine to

machine (IBM has a standard set for types 1-23).  You may now set

any drive type up to # 255; I doubt that any machine actually has

a table this large so you'll probably find garbage in some of

these positions.  (You'll also notice that you can't set type 15;

this is not a valid drive type)

3)  3.5" drives are now supported.  You will also find type '# 5'

through '# 15' available in a bid for future compatibility.

4) Display cards which install their own BIOS (eg, the EGA and VGA

cards) are now supported.  Set EGA/VGA for *any* card which

installs its own BIOS.

5) You will be warned if any of the CMOS error flags are set;  The

possible errors include: Battery is dead, Clock has lost power

(these are NOT the same), RAM checksum is bad, configuration is

bad, memory size is wrong, either hard disk failed to initialize,

or time is incorrect.  Note that some of these are maintained by

the BIOS as part of the power-on test; they may only be correct

after a cold reboot ('Battery dead' is always correct).

6) The system clock is updated every time you change one of the

time or date fields.  The CMOS clock is only updated if you save

the new configuration at the end.  The 'Stand by--Setting clock'

message means the program is waiting to sync with a whole second

to set the clock as accurately as possible. ( Unfortunately, DOS

isn't as picky.  You may find your system clock 1-2 seconds slow

the next time you boot).

7) Features added from the original version:

*      The day of the week is now displayed.  It is maintained by

the BIOS and can't be directly changed, so if your BIOS

can't handle leap years don't blame me. :-)

*      The value of 100ths of seconds is displayed (because it's

there, that's why!).  Again, it can't be changed directly.

*      You can synchronize the clock with the 0 key, no matter

which field you're on.  Synchronizing the clock sets it

backward to the last whole second, so you may have to

fiddle with it a bit.

*      When finished, you will be asked whether to save the

configuration or not.

*      The executable is around 9k smaller, and much faster.

8) Features added from version 2.0 (some are noted above):

*      Warning screen on configuration errors.

*      Corrected a bug in which the display was not restored

properly on exit (that's what I get for trusting the

manual).

*      Added the EGA/VGA display types, 720k, and 1.44 meg drives.

*      Corrected stupid bugs in Hard Drive types.

*      Corrected a bug with date setting; you can no longer set

dates like 2/31/88 by starting with 5/31/88 and changing

the month.

9) I can be reached over Usenet as agollum@engr.uky.edu, or

ukma!ukecc!agollum, or

Kenneth Herron

111 Buchanan Street

Lexington, Kentucky  40508