+-------------+

|  RAMIT!!!   |

+-------------+

If you're  running an  8-bit hard disk controller (generally

designed for  both PCs and ATs) on a 16 bit machine (like an

AT or  AT&T PC6300)  then RAMIT!  may be  able to  help  you

improve your disk transfer speed.

What is RAMIT?

RAMIT is  a program to run the disk

code supplied  with an PC type disk

controller out  of RAM  rather than

from the disk controller's ROM.

What does that do?

By running  out of  RAM rather than

ROM, some  machines can  achieve  a

real  performance   gain  in   disk

transfer time.

Does this apply to

me?

RAMIT can  make a  difference in  a

number of  situations.   To see any

disk performance improvement,

1. You  must be  using a  hard disk

controller which  has its  own BIOS

ROM code.  This includes unmodified

Western Digital,  Mountain, Perstor

and  Plus  [Hardcard]  controllers.

Undoubtedly there are many others.

2. You  must have  a machine  which

runs code  faster from RAM than the

I/O bus.  This includes all 16 bits

machines (AT  types) and many other

compatibles (e.g. AT&T PC6300).

3.  You must be willing to reformat

your hard  disk to  get the benefit

of this  speedup.   Naturally, this

requires a full backup and restore.

Still Interested?

Read on.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

+-----------------------------------+

|  OK, so what does RAMIT! do!!!!   |

+-----------------------------------+

RAMIT! is a TSR which relocates your disk controller ROM into

high-speed RAM and 'fixes up' MSDOS to run the disk code from

RAM.

RAMIT! operates as follows:

1. RAMIT!  verifies that  there  is  a  valid  ROM  BIOS  at

C800:0000.

2. RAMIT! copies the disk ROM contents to RAM.

3. RAMIT!  disassembles the  ROM to  find out  what  it  had

placed in vector locations 4C and 64.  These are the primary

entry points  into the  ROM code.   If  RAMIT! is  unable to

determine  these  values,  then  it  aborts  with  an  error

message.   This is  generally  attributable  to  an  unusual

instruction sequence  used to  'steal' the vectors.  Contact

the author for unusual ROMs.

If you'd rather do the disassembly yourself, the original 4C

and 64  vector addresses  can be  specified on  the  command

line.   This is  the dirty  way to support controllers whose

initialization sequence  is simply  too messy  for automatic

decoding.

If RAMIT! 'knows' about your ROMs, it knows which 4C, 64 and

edit parameters  to use.   RAMIT  calculates a  CRC for  the

resident portion  of a  ROM and scans an internal table.  If

your ROM  is not in the table, contact the author and supply

a floppy disk with the ROM on it.

4. RAMIT!  'fixes' MSDOS or PCDOS by finding where DOS keeps

the vector  originally placed  by  the  disk  ROM  BIOS  and

redirecting it to the RAM copy.

5. RAMIT!  exits, leaving  only a  copy of  the vendor's ROM

code.   All RAMIT! code is removed.  Most unused ROM code is

also removed.   The  amount of  memory taken  depends on the

size and  complexity of the disk controller ROM BIOS.  In no

event will it be more than 8K of code.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

+----------------------------+

|  Well how do I run RAMIT?  |

+----------------------------+

Find out your current performance level

Before seeing whether RAMIT! can make a difference, you must

know how your system performs before RAMIT! is installed.

If your  disk/PC combination  is not explicitly mentioned in

the README.RAM  file in  this archive  (and you  can't  find

another PC close enough), then you may have to experiment to

find the best interleave factor.

SPINTEST is a publicly available utility which computes disk

transfer rate.   It  can be  used to  see when  the  optimum

interleave has been established.

Another program, ATDISK, often distributed as part of the PC

Tech Journal utilities will calculate the transfer rate.

Finally, the  CORETEST program  from Core International will

compute the  transfer rate.   Any  of these  programs can be

used to  determine when  the  optimum  interleave  has  been

found.

First, run SPINTEST to get the transfer rate and interleave.

Then  reduce   the  interleave  until  you  suddenly  see  a

reduction in  the transfer  rate.  At that point you've gone

too far!  Back off by increasing the interleave slightly and

reformat for the last time.

MAKE SURE  THAT YOU'VE  INSTALLED RAMIT  BEFORE RUNNING  ANY

PERFORMANCE TEST.   IF YOU HAVEN'T YOU'RE JUST MEASURING THE

PLAIN  VANILLA  PERFORMANCE  OF  YOUR  CONTROLLER  ROM-BASED

SOFTWARE!!!

The following may serve as an example:

Set IL=6; Spintest says 6 revolutions to read a track.

Set IL=5; Spintest says 5 revolutions to read a track.

Set IL=4; Spintest says 4 revolutions to read a track.

Set IL=3; Spintest says 18 revolutions to read a track.

At this  point, going  to an  interleave of 3 does more

harm than good.  Back off and use an interleave of 4.

An easier  way is  to install  RAMIT and  use one of several

programs which  promise to determine the optimum interleave.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

ILEAVE19.ARC (a public domain program) and Gibson Research's

SPINRITE (definitely  not in the public domain) are two such

programs.  Please note that it is critical to install RAMIT!

before running  these test  programs  otherwise,  you'll  be

getting the best value without RAMIT!.

Testing RAMIT! the first time

At first,  RAMIT! should  be run  in TEST  mode.   This will

indicate whether RAMIT! is likely to work on your machine.

Type

RAMIT /T

and examine  the output.   There  should be  three  messages

indicating that  0000:xxxx  has  been  overwritten.    Don't

worry, in TEST mode the memory is not actually modified.

If RAMIT!  fails or  the messages  don't appear,  check  the

README.RAM file in this archive.  If your drive and computer

isn't on  the list,  be careful.   Things  may very well not

work.

If you  get no error messages, RAMIT! will probably work for

you.

The following  is a sample installation using the /V option.

(Specifying /T assumes /V.)

> RAMIT /V

RAMIT!    [V1.6]  Copyright (c)  1988 Hanover  Systems.

All rights reserved.                   ]  Version

Disk controller vector 1B at C800:0A79 moved to segment

0DB2

Overriding segment address at 0000:1ECB  +  Expect 2

Overriding segment address at 0000:2618  +  Messages

Disk controller vector 25 at C800:09A8 moved to segment

0DB2

Overriding segment address at 0000:261C  ] Expect 1 Msg

Editing segment reference at 06C0        +

Editing segment reference at 06EC        |

Editing segment reference at 0AD2        |

Editing segment reference at 0B59        |  From

Editing segment reference at 0CF4        |  C800

Editing segment reference at 0D55        |  Segment

Editing segment reference at 0E17        |  References

Editing segment reference at 0F19        |

Editing segment reference at 1128        |

Editing segment reference at 125E        +

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

[OEM Disk BIOS] (c) copyrignt XYZZY company.

Installing RAMIT!

WARNING: BEFORE  INSTALLING RAMIT! MAKE SURE THAT YOU HAVE A

FULL  BACKUP  OF  YOUR  HARD  DISKS.    RAMIT!  CHANGES  THE

OPERATION OF  YOUR HARD  DISK CONTROLLER.   WHILE RAMIT! HAS

NOT CAUSED  ANY PROBLEMS  SO FAR, IT HASN'T BEEN RUN ON YOUR

MACHINE!!!!  TAKE PRECAUTIONS AND MAKE A BACKUP!!!!

RAMIT! is installed for normal use by typing

RAMIT

You should  see two  messages: the  standard RAMIT!  version

header and  an identification  for the  disk controller  ROM

BIOS.

Continue normal  operation with  RAMIT! for  a while.    You

should not  see any  performance change.    Just  make  sure

things are still working.

RAMIT! should be placed in the AUTOEXEC.BAT file as near the

top as  possible.   The earlier  it's executed, the more the

AUTOEXEC.BAT programs  will  benefit  from  any  performance

improvement.

Tuning up your disk performance

To see  a real  performance difference, you'll probably have

to reformat  your disk.   This  requires a low-level format,

not just a DOS format (i.e. the FORMAT command).

For Western  Digital customers,  enter  DEBUG.    Old  cards

require that  AX be  set to the interleave.  On newer cards,

the ROM  code prompts  for the interleave.  Start the format

by typing 'G =C800:5'.

OMTI customers can use the OMTIDISK utility.  As part of the

low-level format, you get to specify the interleave.

Many other manufacturers use a similar scheme.  Refer to any

documentation  from  the  disk  controller  manufacturer  to

figure how to perform a low-level format.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

Keep  lowering   the  interleave,   formatting  and  running

SPINTEST until performance suddenly gets worse.  Then do one

more format at the best interleave obtained.

If you've got a unique computer/controller situation, please

drop me  a line  on how  it went.   I'll  include it  in the

README.RAM file  so the next guy doesn't have to do all this

work over again.

RAMIT! Command Format

The syntax for RAMIT! is

RAMIT  [/T]  [/V]  [/Ex]  [/Wfilename]

[/4C:offset]  [/64:offset] [/S:size]

where

/E1  Specifies  EDIT-1  mode.    In  EDIT-1  mode,  the

following instruction sequences get patched:

CMP  word reg,0C800h

If found,  the 0C800h constant is patched to match

the segment where RAMIT! has placed the ROM image.

Some controllers examine their own CS to determine

whether  they're   strapped  for  the  primary  or

secondary addresses.  For example, the OMTI BIOS-7

(Universal) constantly checks its CS with C800.

/E2  Specifies  EDIT-2   mode.      In   EDIT-2   mode,

instructions  matching  either  of  the  following

patterns  are  changed  to  reflect  the  new  RAM

segment:

CMP   byte reg,0C8h   or    POP   AX/BX/CX/DX

JE or JNE                   CMP   xH,0C8h

This is required for the WD Super BIOS.

/V   Specifies VERBOSE  mode.   In VERBOSE mode, RAMIT!

will  print   additional  status  messages  as  it

analyzes and  installs  the  disk  controller  ROM

BIOS.

/S:  Specifies the  [hex] size  of the ROM [in 512 byte

blocks].   This value  overrides that given in the

ROM itself.   Some  controllers 'lie'  about their

true size.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

/T   Specifies TEST  mode.   In TEST  mode, RAMIT! will

perform its analysis BUT WILL NOT MAKE ANY CHANGES

TO DOS,  OR INSTALL  A RAM  COPY OF  THE ROM BIOS.

TEST mode  is  provided  so  that  RAMIT!  can  be

checked on  new controllers  or versions  of  DOS.

TEST mode forces VERBOSE mode.

A successful  TEST will produce no error messages,

and identify THREE overwritten locations;  two for

vector 1B and another for vector 25.

If this  is not  the case,  RAMIT  may  not  work.

Contact the author for more instructions.

/W   Indicates that  the disk controller ROM BIOS is to

be  written  out  to  the  file  specified.    The

'filename' immediately follows the /W; for example

'/WDISK.ROM'.

This feature is provided so RAMIT! can capture new

or  updated   disk  controller   ROMs  for  future

support.     If  you   have  a  non-standard  disk

controller, and  it won't install with RAMIT!, use

the /W  option to  make a disk copy and send it to

me.   I'll try  to extend  RAMIT!  for  your  disk

controller.

/4C: Explicitly  specifies an offset which the BIOS had

stuffed into  vector location  4C.    Rather  than

disassembling the  ROM code,  this  value  can  be

specified on the command line.

/64: Explicitly  specifies an offset which the BIOS had

stuffed into  vector location  64.    Rather  than

disassembling the  ROM code,  this  value  can  be

specified on the command line.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

+---------------------------------------------------+

| A Quick Tutorial on hard disk controller software |

+---------------------------------------------------+

Getting started.

When your  computer is  started, on-board  software in  Read

Only Memory (ROM) is started.  This software, called the ROM

BIOS, performs  a cursory  check of  the  machine  and  then

initializes all  the devices  it knows about (CRT, keyboard,

floppy etc).

If you  have the  manufacturer's hard disk controller or the

manufacturer can  support the third party controller in your

machine, the ROM BIOS will take control of it.  If, however,

you're using  another board,  the original  ROM BIOS doesn't

know how to handle it.

So that  you computer doesn't just stare at you, not knowing

how to  boot from  your hard  disk, IBM  set up  a procedure

whereby anybody  could gain  control automatically  from the

ROM BIOS and provide their own hard disk software.

To quote from the IBM Technical Reference,

"The ROM  BIOS provides a facility to integrate adapter

cards with  on-board ROM  code into the system.  During

POST  [Power  On  Self  Test],  interrupt  vectors  are

established for  the BIOS  calls.   After  the  default

vectors are in place, a scan for additional ROM modules

takes place.   At  this point,  a ROM  routine  on  the

adapter  card   may  gain  control    The  routine  may

establish or  intercept vectors to hook themselves into

the system.

"The absolute addresses hex C8000 through hex F4000 are

scanned in  2K blocks in search of a valid adapter card

ROM.  A valid ROM is defined as follows:

"Byte 0: Hex 55.

"Byte 1: Hex AA.

"Byte 2:  A length indicator representing the number of

512-byte blocks in the ROM (length/512).   ...

"When the  POST identifies  a valid  ROM, it does a far

call to  byte 3  of the ROM (which should be executable

code).   The adapter  card may now perform its power-on

initialization tasks.   The  feature ROM  should return

control to the BIOS routines by executing a far return.

All of  the cards  supported by RAMIT! use this technique to

initialize the  disk drive  itself, and  'steal' vectors  in

order to gain control when a disk command is required.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

Doing Disk I/O.

When DOS  or a program want to do disk I/O, it loads up some

registers with  the I/O parameters and executes an 'INT 13h'

instruction.  The CPU ends up calling whatever routine whose

address is in memory for 'vector 13'.

At  power-on  time,  the  ROM  BIOS  puts  its  floppy  disk

handler's address in this memory location.

If external  ROM  code  on  the  disk  controller  board  is

present, at  initialization time  it will  copy the  default

(ROM BIOS)  address to a private location (normally location

100h) and  supply its  own  address  for  vector  13.    The

controller's ROM  will then  receive control  after the 'INT

13h' instruction.   It will examine the parameters passed in

the registers.   If  the request  is not intended for one of

its disks,  then the  controller  ROM  code  will  pass  the

request along  to whatever  address it  found for  vector 13

originally.  This would normally be the ROM BIOS floppy disk

handler.

In this way, the disk controller's ROM code gets a chance to

process every  disk I/O request.  This provides a simple way

for anyone  to make  a disk controller which works in nearly

all PC-compatibles.

Of course,  there's no  reason why  the hard disk controller

would be  the only piece of code to take over the vector 13!

In fact,  MSDOS and  PCDOS both  do this  regularly.    They

provide a  handler to  scan all  disk I/O  for errors.   The

message

I/O Error

Abort, Retry or Ignore?

is a  DOS error trapped in this way.  Other programs such as

disk  caches   will  also   trap  this  vector.    The  only

requirement is  that  each  intercepting  program  which  is

unable to service a request fully must pass it along via the

vector 13 handler which was in place before the intercepting

program took over.

This all seems very logical and is, in fact, a very powerful

mechanism  to  handle  disk  I/O  in  a  device  independent

fashion.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

Ok, then where's the problem?

-----------------------------

The speed difference between system RAM and PC-bus ROM.

The code  which resides on the disk controller card is 'out'

on the  PC bus.   It is as 'reachable' and executable as any

code on in RAM or on the mother-board ROMs (where the system

ROM BIOS lives).

For a  16 bit  machine,  there  is  an  enormous  difference

between on-board  RAM/ROM and  PC bus ROM:  each instruction

fetched from  the on-board RAM/ROM is read in a 16 bit chunk

at the  machine's maximum  speed.   Each instruction fetched

from the  PC bus  is  fetched  in  two  separate  operations

because the PC bus is only 8 bits wide.  In addition, the PC

bus often operates at one half the speed of the main memory,

or less.  Consequently, it can take anywhere from two to ten

times as  long to execute code based on the PC bus as on the

mother-board.

For example,  suppose you have a Western Digital PC (not AT)

controller on  an AT type clone running at 12 Mhz.  First, a

16 bit  instruction fetch  must be  converted into two 8 bit

PC-bus fetches.   This  automatically doubles  any  request.

Next, our clone manufacturer tried to be very compatible, so

his PC-bus runs at 6MHz, just like the original IBM AT.  Add

in a  few wait  states and we will end up taking three times

as long to execute a simple instruction from PC-bus ROM than

from mother-board ROM or RAM.

So what if it's slower, the disk is slower still?

-------------------------------------------------

Up to  a point  this is  true.   Unfortunately, if  the disk

controller software  cannot get  commands out to the disk in

time, it  may miss  a whole  revolution.  At that point your

computer is  quite useless,  waiting for  the disk  to  come

around again.   The  proper spacing  of data  around a  disk

track is  determined by the disk 'interleave'.  If you don't

know what interleaving is or why it's so important, read the

next section.

The best  interleave is  determined by the raw transfer rate

of the  machine and its hardware.  You can move data only so

fast from  disk to  memory.   It is  also determined  by the

speed  of   the  controlling  software  which  controls  the

hardware.   If you can speed the disk controller's software,

you won't  need as  much  time  to  execute  those  critical

functions and  you  may  be  able  to  run  with  a  smaller

interleave.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

By way  of example,  I have  a AT&T  PC6300.  The 'generous'

retailer who  formatted my  disk used  the wrong interleave.

Consequently, it  took 18  revolutions to  read  a  mere  17

sectors from  my hard  disk!!   After reading  one sector, I

wasn't getting  ready to  read the next in time, so I had to

wait a  whole revolution!.   (PS.  there are  17 sectors per

track  on   most  hard  disks;  26  per  track  for  an  RLL

controller.)

Thanks to  a program  by Steve Gibson, SPINTEST, I found the

ideal interleave  was 6.  After reformatting my disk, it now

took 'only'  6 revolutions  to read  a track,  a three  fold

improvement which  was immediately  noticeable when  loading

large programs.

Trying any interleave lower ('better') than 6 put me back to

17 or 18 revolutions to read a track.

When I  ran the  Western Digital  controller code from high-

speed RAM  rather than  the slow-speed PC bus, I was able to

use an  interleave of 4.  This allowed a track to be read in

only 4  revolutions.  Without any additional hardware, I was

able to  read raw  data 50%  faster just by running from RAM

over ROM.

With the  OMTI RLL  (5527) controller,  I was able to reduce

the interleave from 6 to 3 on my AT&T machine.

The only trick was getting my machine to run from RAM rather

than ROM.  That's what RAMIT! does.

Great!  Give me an example!

---------------------------

For WD  controllers (with  the simple BIOS), leaving the ROM

BIOS chip in, and W3 still strapped, I would use:

RAMIT

This figures  everything out for itself and installs the ROM

BIOS into RAM.

For controllers  I have examined, RAMIT! will be able to run

without parameters.    I  have  made  entries  into  RAMIT!s

internal table  so it knows the /4C, /64 and /Ex parameters.

Unfortunately, if your ROM differs by even a single bit from

the one I examined, RAMIT! will not recognize it by its CRC.

In this case, you'll have to rely on the disassembler or the

command line overrides.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

For the  OMTI 5527 RLL controller with the universal BIOS, I

use

RAMIT  /E1  /4c:a79  /64:9a8

The /E1  (edit) converts C800 constants to whatever-segment-

the-ROM-has-been-copied-to;   the /4c:  and  /64:  give  the

original offsets  that  the  BIOS  had  stuffed  into  those

respective vectors.

For the WD controller with the Super Bios, I use

RAMIT  /E2  /4c:31a  /64:26d

The /E2  (edit) converts  the  tests  for  segment  C800  to

whatever-segment-the-ROM-has-been-copied-to;   the /4c:  and

/64: give  the original  offsets that  the BIOS  had stuffed

into the respective vectors.

Alas, the  WD RLL  controller is much more efficient because

it has  a much  larger  on-board  buffer.    This  evidently

reduces  the   CPU  interaction   and  thereby  lessens  the

opportunity  to  improve  the  controller's  performance  by

running from RAM.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

+-----------------------------------------+

|  A quick tutorial on disk interleaving  |

+-----------------------------------------+

What is interleaving?

Interleaving is  the  technique  where  consecutive  logical

sectors are places several physical sectors apart around the

track.   The interleave  factor is  the  number  of  sectors

between consecutive  logical sectors.   For  example,  a  10

sector track  with an  interleave of  1  would  arrange  its

sectors

1,2,3,4,5,6,7,8,9,10

while one with an interleave of 3 would be arranged

1,8,5,2,9,6,3,10,7,4

Why bother interleaving?

Controllers interleave  because they cannot transfer data to

memory as  fast as  they can  read it  off the  disk.   Some

controllers won't  transfer anything  unless the error check

codes are correct which cannot be determined until after the

entire sector is read.

If the  sectors were  placed sequentially  on disk,    after

reading one sector,there  would   not  be  enough  time  to

transfer it  before the  next one  came along under the read

heads.   (Obviously, the  disk isn't  going to stop spinning

just because  the  controller  isn't  ready.)    By  putting

sequential sectors  the proper  distance apart,  the desired

sector is just about to be read when the controller is ready

to process it.

What is wrong with having the wrong interleave?

With the  proper interleave,  after reading and transferring

sector 'n',  sector 'n+1'  is just about to come by.  If the

interleave is  too large,  we'll have  to skip over at least

one sector  which is  not 'n+1'.   This  will waste whatever

time it  takes to  skip over that sector.  If the interleave

is too  small, we've  already missed  sector 'n+1'.  We must

wait a  whole revolution  of the  disk before we get another

shot at  it.  If we were to read all 17 sectors (on a normal

disk), we'd require 17 revolutions.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

What should the PC6300 interleave be?

If you  have the  standard  [MFM]  WD  disk  controller,  an

interleave of 6 works best.  This is readily verified with a

number of public domain disk test programs.

An interleave of 6 requires 6 revolutions to read one track.

If you  have the  OMTI RLL  controller, an  interleave of  4

works best.

What should an AT interleave be?

If you  have the  standard WD disk controller, an interleave

of 3  works best.   This  is the  standard low-level  format

value.

How do I change my interleave?

Unfortunately, the  only way  to change this [at the moment]

is  to   perform  a   low-level  reformat   of  your   disk.

Instructions vary  according to  manufacturer.    For  many,

enter  debug   and  jump   to  location  C800:0005.    After

performing a  low-level format,  a  regular  DOS  format  is

required.

How do I know how my interleave works?

An excellent utility available at bulletin boards across the

country is  SPINTEST.   This program  was written  by  Steve

Gibson and reports how many revolutions are required to read

a track.   Some experimentation is required to determine the

optimal interleave  value.   As I  get reports for different

controller/computer  combinations,  I'll  put  them  in  the

associated README.RAM file.

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988

How is RAMIT! distributed!

RAMIT! is distributed as share-ware.  RAMIT! remains the

copyrighted product of Hanover Systems, Newtown CT.  Private

users are urged to register their copies for $25.  Commercial

users must register.  OEM and site licensing is available.

Registering will guarantee continued access to RAMIT! and

continued upgrades free of charge.

Further inquiries, ROM images or registrations may be directed to

the author

Christopher Smith

Hanover Systems

19 Tunnell Road

Newtown, CT. 06470-1242

(203) 426-0024

RAMIT!                  Copyright (c) Hanover Systems, 1987, 1988